Section 8 Inspection Checklist Pre inspection self check based on HUD NSPIRE standards

| ADDRESS AND UNIT [Address and unit] | INSPECTION DATE [Inspection date] |
| --- | --- |

| CHECKED BY [Checked by] | YEAR BUILT [Year built] |
| --- | --- |

SMOKE AND CARBON MONOXIDE ALARMS

☐ Smoke alarm on every level, inside each bedroom and within 21 feet of each bedroom door; each one tested and working

☐ Carbon monoxide alarm where the housing authority requires it (for example, fuel burning appliances or a fireplace)

HEAT AND WATER

☐ A permanently installed heat source works (no unvented gas, oil or kerosene space heaters)

☐ Furnace or boiler exhaust vent connected and not blocked; combustion cover and gas shutoff in place

☐ Hot and cold running water at the kitchen sink and in the bathroom

☐ Water heater: no leaks, pressure relief valve and discharge pipe in place, vent connected

ELECTRICAL

☐ Outlets within 6 feet of a sink, tub or other water source are GFCI protected and trip and reset

☐ No exposed wires, missing cover plates or open spaces in the breaker panel

☐ Every habitable room has 2 working outlets, or 1 outlet and a permanent light

☐ Permanent light fixture works in the kitchen and in each bathroom

KITCHEN

☐ Stove or range works, including all burners and the oven

☐ Refrigerator cools and the door seals

☐ Sink drains, no leaks, with space to prepare and store food

BATHROOM

☐ Toilet flushes, fills and is not loose

☐ Sink and tub or shower work and drain; no leaks

☐ Window that opens or a working exhaust fan

DOORS, WINDOWS AND EXITS

☐ Entry doors close and lock

☐ Windows open, close and lock where designed to; no broken glass

☐ Exits and bedroom windows used for escape are not blocked

WALLS, FLOORS, STAIRS AND RAILS

☐ No holes or loose material in walls, ceilings or floors

☐ Stairs have working handrails; guardrails where there is a drop

☐ No trip hazards or sharp edges

☐ Dryer exhaust duct connected (if there is a dryer)

LEAKS, MOLD AND PESTS

☐ No water, gas or sewage leaks

☐ No mold like growth

☐ No signs of roaches, rodents or bed bugs

OUTSIDE AND LEAD PAINT

☐ Address visible; steps, walkways and railings safe; outside outlets GFCI protected

☐ Built before 1978: no chipped, peeling or deteriorated paint

BEFORE THE INSPECTOR ARRIVES

☐ Owner paid utilities on, appliances in place, and the tenant told the date and time

ITEMS TO FIX AND DATE FIXED [Items to fix and date fixed]

Instructions for the landlord Keep this page. Do not send it to the tenant.

HOW TO USE IT

- Walk the unit a week or two before the inspection and tick each item that passes.

- Fix anything not ticked, and write the date you fixed it.

- Make sure the tenant knows the inspection date and time.

DEADLINES AND STANDARDS

- Life threatening deficiencies must be fixed within 24 hours of notice; most others within 30 days (HUD Notice PIH 2026-18).

- Housing authorities must use NSPIRE for voucher inspections by February 1, 2027, and may use HQS until then.

- Ask your housing authority which standard and checklist it uses. This is not an official HUD form.

FREE GUIDE

simplerentledger.com/section-8-inspection-checklist/